VMware Virtual Lab

3-VM Network Setup Guide

Client  |  DNS/DHCP Server  |  Web Server

Two Isolated Networks + Bridged Internet Access

VMnet2 (192.168.10.0/24)  —  Client LAN

VMnet3 (172.16.0.0/24)  —  Server Network

Bridged Adapter (DHCP from host network)  —  Internet Access on all VMs

Operating System: Ubuntu Server 22.04 LTS / Ubuntu Desktop

Hypervisor: VMware Workstation Pro

# 1. Overview

This guide walks you through setting up a virtual lab with three VMs on VMware, connected via two isolated networks. Each VM also has a Bridged network adapter for internet access, allowing you to download packages and updates. The DNS/DHCP server bridges both internal networks and acts as a router, allowing the client to reach the web server.

## 1.1 Network Architecture

VMnet2 (192.168.10.0/24) connects the Client and the DNS/DHCP Server.

VMnet3 (172.16.0.0/24) connects the DNS/DHCP Server and the Web Server.

Bridged Adapter is added to every VM to provide internet connectivity for downloading packages. It obtains an IP via DHCP from your physical/home network.

The DNS/DHCP Server has three NICs: one on each internal network plus the bridged adapter. The Client and Web Server each have two NICs: one internal and one bridged.

| Component | Internal NIC(s) | Bridged NIC | Internal IP(s) |
| --- | --- | --- | --- |
| Client VM | VMnet2 | Bridged (DHCP) | DHCP from server |
| DNS/DHCP Server | VMnet2 + VMnet3 | Bridged (DHCP) | 192.168.10.1 / 172.16.0.1 |
| Web Server | VMnet3 | Bridged (DHCP) | 172.16.0.10 |

## 1.2 Prerequisites

- VMware Workstation Pro (version 16 or later recommended)

- Ubuntu Server 22.04 LTS ISO (for DNS/DHCP and Web Server)

- Ubuntu Desktop 22.04 ISO or Windows 10/11 ISO (for Client)

- Minimum 8 GB RAM on host (2 GB per VM recommended)

- At least 60 GB free disk space on host

- A working internet connection on the host machine (the Bridged adapter shares this with VMs)

# 2. Create Virtual Networks in VMware Workstation Pro

Before creating any VMs, you need to configure two isolated virtual networks using VMware Workstation Pro’s Virtual Network Editor. These networks will not use VMware’s built-in DHCP — you will run your own DHCP server. Each VM will also use a Bridged adapter for internet access.

## 2.1 Open the Virtual Network Editor

- Open VMware Workstation Pro and go to Edit → Virtual Network Editor

- Click Change Settings at the bottom right (requires administrator/root privileges). A UAC prompt may appear — click Yes.

## 2.2 Create VMnet2 (Client LAN)

- Click Add Network...

- Select VMnet2 from the dropdown and click OK.

- Select VMnet2 in the list and set the type to Host-only

- Set Subnet IP to 192.168.10.0 and Subnet mask to 255.255.255.0

- Uncheck "Use local DHCP service to distribute IP addresses to VMs"

## 2.3 Create VMnet3 (Server Network)

- Click Add Network...

- Select VMnet3 from the dropdown and click OK.

- Select VMnet3 in the list and set the type to Host-only

- Set Subnet IP to 172.16.0.0 and Subnet mask to 255.255.255.0

- Uncheck "Use local DHCP service to distribute IP addresses to VMs"

## 2.4 Summary of Virtual Network Editor Settings

| Setting | VMnet2 (Client LAN) | VMnet3 (Server Network) |
| --- | --- | --- |
| Type | Host-only | Host-only |
| Subnet IP | 192.168.10.0 | 172.16.0.0 |
| Subnet Mask | 255.255.255.0 | 255.255.255.0 |
| Use local DHCP | Unchecked | Unchecked |
| Connect host adapter | Optional | Optional |

- Click Apply and then OK to save all changes.

Note: Disabling VMware’s built-in DHCP is critical. If left enabled, it will conflict with your DHCP server and cause IP assignment issues.

## 2.5 About the Bridged Adapter (Internet Access)

The Bridged adapter does not need to be created in the Virtual Network Editor — it is available by default in VMware Workstation Pro. When you assign a NIC to "Bridged" mode, VMware connects it directly to your host’s physical network adapter. The VM then gets an IP from your home/office router via DHCP, giving it full internet access for downloading packages.

- Each VM will have one NIC set to Bridged (Automatic) for internet.

- The Bridged NIC will get an IP from your physical network’s DHCP (e.g., 192.168.1.x from your home router).

- This NIC is used only for downloading packages — it does not participate in the lab’s internal networking.

- Leave "Replicate physical network connection state" unchecked — it is not needed for this lab.

Tip: You can optionally disable the Bridged adapter after all packages are installed to fully isolate the lab environment.

# 3. Create the Client VM

The Client VM is the first machine you will set up. It has two NICs: one on VMnet2 (Client LAN) for internal communication, and one Bridged adapter for internet access and package downloads.

## 3.1 Create the Virtual Machine

- In VMware Workstation Pro, go to File → New Virtual Machine...

- Select Typical (recommended) and click Next.

- Select Installer disc image file (iso) and browse to your Ubuntu Desktop 22.04 ISO (or Windows 10/11 ISO). Click Next.

- Set the VM name to Client-VM. Choose a storage location if needed. Click Next.

- Set disk size to 20 GB and select "Store virtual disk as a single file". Click Next.

- On the summary screen, click Customize Hardware... before finishing.

- Set Memory to 2048 MB (2 GB) and Processors to 2.

- Click Close, then Finish.

## 3.2 Configure Network Adapters (2 NICs)

Before powering on the VM, configure the network adapters.

### NIC 1 — Internal (Client LAN)

- Right-click the VM in the library and select Settings...

- Select Network Adapter in the hardware list.

- Under "Network connection", select Custom: Specific virtual network

- Choose VMnet2 from the dropdown.

### NIC 2 — Internet Access (Bridged)

- Click the Add... button at the bottom of the Settings window.

- Select Network Adapter and click Finish.

- The new adapter (Network Adapter 2) appears. Select it and set it to Bridged: Connected directly to the physical network

- Leave "Replicate physical network connection state" unchecked.

- Click OK to save.

| NIC | VMware Network | IP Address | Purpose |
| --- | --- | --- | --- |
| NIC 1 (ens33) | Custom: VMnet2 | DHCP from your server | Internal Client LAN |
| NIC 2 (ens36) | Bridged | DHCP from home router | Internet / package downloads |

## 3.3 Install the Operating System

- Power on the VM and install Ubuntu Desktop (or Windows) following the standard installation wizard.

- The Bridged adapter will provide internet during installation, making the process smoother.

- During network configuration, select DHCP (this is the default).

- Complete the installation and reboot.

## 3.4 Configure Networking on the Client

After installation, configure the internal NIC for DHCP from your server, and the bridged NIC for internet access.

### Ubuntu Desktop

Edit the Netplan configuration file:

sudo nano /etc/netplan/00-installer-config.yaml

Set the contents to:

network:

version: 2

ethernets:

ens33:                        # NIC 1 - Internal (VMnet2)

dhcp4: true

ens36:                        # NIC 2 - Bridged (Internet)

dhcp4: true

Apply the configuration:

sudo netplan apply

Verify both interfaces are active:

ip addr show

# ens33 should get IP from your DHCP server (192.168.10.x)

# ens36 should get IP from your home router (e.g., 192.168.1.x)

### Windows Client

- Open Settings → Network & Internet → Ethernet

- Both adapters should be set to Automatic (DHCP).

- The Bridged adapter will auto-connect to the internet.

### Test Internet Connectivity

ping -c 4 google.com

# Should succeed via the Bridged adapter

Note: The internal NIC (ens33) will not get an IP address until the DNS/DHCP server is configured and running in Section 4. The Bridged NIC (ens36) will work immediately for internet access.

# 4. Create the DNS/DHCP Server

The DNS/DHCP Server is the central VM that bridges both internal networks. It has three network interfaces: one on VMnet2 (Client LAN), one on VMnet3 (Server Network), and one Bridged adapter for internet access. It provides DHCP and DNS services to the client and routes traffic between the two internal networks.

## 4.1 Create the Virtual Machine

- Go to File → New Virtual Machine...

- Select Typical (recommended) and click Next.

- Select Installer disc image file (iso) and browse to your Ubuntu Server 22.04 ISO. Click Next.

- Set the VM name to DNS-DHCP-Server. Click Next.

- Set disk size to 20 GB. Click Next.

- Click Customize Hardware... and set Memory to 2048 MB and Processors to 2.

- Click Close, then Finish.

## 4.2 Configure Network Adapters (3 NICs)

This VM requires three network adapters. Right-click the VM and select Settings...

### NIC 1 — Client LAN

- Select Network Adapter in the hardware list.

- Set to Custom: Specific virtual network → VMnet2

### NIC 2 — Server Network

- Click Add... → Network Adapter → Finish

- Select Network Adapter 2 and set to Custom: Specific virtual network → VMnet3

### NIC 3 — Internet Access (Bridged)

- Click Add... → Network Adapter → Finish

- Select Network Adapter 3 and set to Bridged: Connected directly to the physical network

- Leave "Replicate physical network connection state" unchecked.

- Click OK to save all settings.

| NIC | VMware Network | IP Address | Purpose |
| --- | --- | --- | --- |
| NIC 1 (ens33) | Custom: VMnet2 | 192.168.10.1 (static) | Client LAN |
| NIC 2 (ens36) | Custom: VMnet3 | 172.16.0.1 (static) | Server Network |
| NIC 3 (ens37) | Bridged | DHCP from home router | Internet access |

## 4.3 Install Ubuntu Server

- Power on the VM and install Ubuntu Server 22.04.

- During the network setup step, you can skip configuration (it will be done manually).

- Install OpenSSH server when prompted (useful for remote management).

- Complete the installation and reboot.

## 4.4 Configure Static IP Addresses

After logging in, configure both internal network interfaces with static IPs. The Bridged adapter (ens37) uses DHCP for internet access.

Edit the Netplan configuration:

sudo nano /etc/netplan/00-installer-config.yaml

Replace the contents with:

network:

version: 2

ethernets:

ens33:                        # NIC 1 - Client LAN (VMnet2)

addresses:

- 192.168.10.1/24

ens36:                        # NIC 2 - Server Network (VMnet3)

addresses:

- 172.16.0.1/24

ens37:                        # NIC 3 - Bridged (Internet)

dhcp4: true

Apply the configuration:

sudo netplan apply

Verify all three interfaces are up:

ip addr show

# ens33 should show 192.168.10.1

# ens36 should show 172.16.0.1

# ens37 should show an IP from your home router (e.g., 192.168.1.x)

Test internet connectivity:

ping -c 4 google.com

Tip: If your NIC names differ (e.g., enp0s3, eth0), run "ip link show" to identify them.

## 4.5 Enable IP Forwarding (Routing)

Since this server bridges two networks, IP forwarding must be enabled so traffic can pass between VMnet2 and VMnet3.

echo "net.ipv4.ip_forward=1" | sudo tee -a /etc/sysctl.conf

sudo sysctl -p

Verify:

cat /proc/sys/net/ipv4/ip_forward    # Should output: 1

## 4.6 Install and Configure DHCP Server

Install the ISC DHCP server:

sudo apt update

sudo apt install isc-dhcp-server -y

### Configure DHCP

Edit the main DHCP configuration file:

sudo nano /etc/dhcp/dhcpd.conf

Replace or add the following configuration:

# Global options

option domain-name "lab.local";

option domain-name-servers 192.168.10.1;

default-lease-time 600;

max-lease-time 7200;

authoritative;

# Client LAN subnet - serves DHCP addresses

subnet 192.168.10.0 netmask 255.255.255.0 {

range 192.168.10.100 192.168.10.200;

option routers 192.168.10.1;

option domain-name-servers 192.168.10.1;

}

# Server Network subnet - declared but no DHCP pool

subnet 172.16.0.0 netmask 255.255.255.0 {

}

### Set the Listening Interface

Tell the DHCP server to listen only on the Client LAN interface:

sudo nano /etc/default/isc-dhcp-server

Set:

INTERFACESv4="ens33"

### Start the DHCP Service

sudo systemctl restart isc-dhcp-server

sudo systemctl enable isc-dhcp-server

sudo systemctl status isc-dhcp-server

Note: The subnet 172.16.0.0 declaration (with no range) is required. Without it, the DHCP server will fail to start because it detects an interface on a subnet without a matching declaration. You do not need to declare the Bridged adapter’s subnet — the DHCP server only listens on ens33 as configured above.

## 4.7 Install and Configure DNS Server (BIND9)

Install BIND9:

sudo apt install bind9 bind9-utils -y

### Configure BIND Options

Edit the main options file:

sudo nano /etc/bind/named.conf.options

Replace with:

options {

directory "/var/cache/bind";

listen-on { 192.168.10.1; 172.16.0.1; 127.0.0.1; };

allow-query { any; };

forwarders {

8.8.8.8;

8.8.4.4;

};

recursion yes;

};

### Create the Forward Zone

Add the zone declaration:

sudo nano /etc/bind/named.conf.local

Add:

zone "lab.local" {

type master;

file "/etc/bind/zones/db.lab.local";

};

### Create the Zone File

sudo mkdir -p /etc/bind/zones

sudo nano /etc/bind/zones/db.lab.local

Add the following DNS records:

$TTL    604800

@       IN      SOA     ns1.lab.local. admin.lab.local. (

3         ; Serial

604800         ; Refresh

86400         ; Retry

2419200         ; Expire

604800 )       ; Negative Cache TTL

; Name server record

@       IN      NS      ns1.lab.local.

; A records

ns1         IN      A       192.168.10.1

www         IN      A       172.16.0.10

webserver   IN      A       172.16.0.10

### Validate and Start DNS

sudo named-checkconf

sudo named-checkzone lab.local /etc/bind/zones/db.lab.local

sudo systemctl restart bind9

sudo systemctl enable bind9

sudo systemctl status bind9

## 4.8 Verify the DNS/DHCP Server

Run these checks on the DNS/DHCP server itself:

# Check all three interfaces have correct IPs

ip addr show ens33       # Should show 192.168.10.1

ip addr show ens36       # Should show 172.16.0.1

ip addr show ens37       # Should show IP from home router

# Check IP forwarding is enabled

cat /proc/sys/net/ipv4/ip_forward   # Should show 1

# Check internet connectivity

ping -c 4 google.com    # Should succeed via Bridged

# Check DHCP is running

sudo systemctl status isc-dhcp-server

# Check DNS is running

sudo systemctl status bind9

# Test DNS locally

nslookup www.lab.local 127.0.0.1

# 5. Create the Web Server

The Web Server has two NICs: one on VMnet3 (Server Network) for internal communication, and one Bridged adapter for internet access to download packages. It hosts an Apache web server that will be accessible to the client through the DNS/DHCP server acting as a router.

## 5.1 Create the Virtual Machine

- Go to File → New Virtual Machine...

- Select Typical (recommended) and click Next.

- Select Installer disc image file (iso) and browse to your Ubuntu Server 22.04 ISO. Click Next.

- Set the VM name to Web-Server. Click Next.

- Set disk size to 20 GB. Click Next.

- Click Customize Hardware... and set Memory to 2048 MB and Processors to 2.

- Click Close, then Finish.

## 5.2 Configure Network Adapters (2 NICs)

Right-click the VM and select Settings...

### NIC 1 — Server Network (Internal)

- Select Network Adapter in the hardware list.

- Set to Custom: Specific virtual network → VMnet3

### NIC 2 — Internet Access (Bridged)

- Click Add... → Network Adapter → Finish

- Select Network Adapter 2 and set to Bridged: Connected directly to the physical network

- Leave "Replicate physical network connection state" unchecked.

- Click OK to save.

| NIC | VMware Network | IP Address | Purpose |
| --- | --- | --- | --- |
| NIC 1 (ens33) | Custom: VMnet3 | 172.16.0.10 (static) | Server Network (internal) |
| NIC 2 (ens36) | Bridged | DHCP from home router | Internet / package downloads |

## 5.3 Install Ubuntu Server

- Power on and install Ubuntu Server 22.04.

- The Bridged adapter will provide internet during installation.

- Install OpenSSH server when prompted.

- Complete installation and reboot.

## 5.4 Configure Network Interfaces

Configure the internal NIC with a static IP and the Bridged NIC for internet access.

Edit Netplan:

sudo nano /etc/netplan/00-installer-config.yaml

Set the following:

network:

version: 2

ethernets:

ens33:                        # NIC 1 - Server Network (VMnet3)

addresses:

- 172.16.0.10/24

routes:

- to: 192.168.10.0/24

via: 172.16.0.1

nameservers:

addresses:

- 172.16.0.1

ens36:                        # NIC 2 - Bridged (Internet)

dhcp4: true

Apply the configuration:

sudo netplan apply

Verify both interfaces:

ip addr show

# ens33 should show 172.16.0.10

# ens36 should show IP from home router

Test internet connectivity:

ping -c 4 google.com

Note: The route to 192.168.10.0/24 via 172.16.0.1 is essential. Without it, the web server will not be able to send responses back to the client network.

## 5.5 Install and Configure Apache

sudo apt update

sudo apt install apache2 -y

Create a custom homepage:

echo "<h1>Hello from Web Server - lab.local</h1>" | sudo tee /var/www/html/index.html

Enable and start Apache:

sudo systemctl enable apache2

sudo systemctl start apache2

sudo systemctl status apache2

## 5.6 Configure Firewall (Optional)

If UFW is enabled, allow HTTP traffic:

sudo ufw allow 80/tcp

sudo ufw allow 22/tcp

sudo ufw enable

sudo ufw status

## 5.7 Verify the Web Server

# Check IP addresses

ip addr show ens33          # Should show 172.16.0.10

ip addr show ens36          # Should show IP from home router

# Test internet connectivity

ping -c 4 google.com        # Should succeed via Bridged

# Ping the DNS/DHCP server

ping 172.16.0.1             # Should succeed

# Test DNS resolution

nslookup www.lab.local 172.16.0.1

# Test Apache locally

curl http://localhost

# 6. Complete Verification and Testing

Now that all three VMs are configured, run these tests from the Client VM to verify the entire setup works end-to-end.

## 6.1 Verify DHCP (Client Gets an IP)

On the Client VM:

ip addr show ens33

# Expected: an IP in the range 192.168.10.100 - 192.168.10.200

If no IP is assigned, try releasing and renewing:

sudo dhclient -r ens33

sudo dhclient ens33

## 6.2 Verify DNS Resolution

nslookup www.lab.local

# Expected output:

# Server:    192.168.10.1

# Address:   192.168.10.1#53

# Name:      www.lab.local

# Address:   172.16.0.10

## 6.3 Test Network Connectivity

# Ping the DNS/DHCP server (same network)

ping -c 4 192.168.10.1

# Ping the Web Server (different network, routed)

ping -c 4 172.16.0.10

## 6.4 Access the Web Server

# Using curl from terminal

curl http://www.lab.local

# Expected: <h1>Hello from Web Server - lab.local</h1>

# Or open a browser and navigate to:

# http://www.lab.local

| Test | Command | Expected Result |
| --- | --- | --- |
| DHCP Lease | ip addr show | IP in 192.168.10.100-200 range |
| DNS Resolution | nslookup www.lab.local | Resolves to 172.16.0.10 |
| Ping DNS/DHCP | ping 192.168.10.1 | Reply received |
| Ping Web Server | ping 172.16.0.10 | Reply received (routed) |
| Web Access | curl http://www.lab.local | HTML page displayed |

# 7. Troubleshooting

## 7.1 Client Not Receiving an IP Address

- Verify the DHCP server is running: sudo systemctl status isc-dhcp-server

- Check the DHCP server logs: sudo journalctl -u isc-dhcp-server -f

- Confirm the Client NIC is connected to VMnet2 in VM Settings.

- Ensure VMware’s built-in DHCP for VMnet2 is disabled.

- Restart networking on the client: sudo netplan apply

## 7.2 DNS Not Resolving

- Verify BIND9 is running: sudo systemctl status bind9

- Check for configuration errors: sudo named-checkconf

- Validate the zone file: sudo named-checkzone lab.local /etc/bind/zones/db.lab.local

- Check DNS logs: sudo journalctl -u bind9 -f

- Test directly: dig @192.168.10.1 www.lab.local

## 7.3 Client Cannot Ping the Web Server

- Verify IP forwarding is enabled on the DNS/DHCP server: cat /proc/sys/net/ipv4/ip_forward

- Check the routing table on the DNS/DHCP server: ip route

- Verify the web server has the correct route back: ip route show (should include 192.168.10.0/24 via 172.16.0.1)

- Check for firewall rules blocking traffic: sudo iptables -L -n

- Disable UFW temporarily for testing: sudo ufw disable

## 7.4 Web Page Not Loading

- Verify Apache is running on the web server: sudo systemctl status apache2

- Test Apache locally on the web server: curl http://localhost

- Check if port 80 is open: sudo ss -tlnp | grep :80

- Verify firewall allows HTTP: sudo ufw status

## 7.5 NIC Names Are Different

With multiple NICs, your interface names may differ from ens33/ens36/ens37. Use these commands to identify them:

ip link show              # List all interfaces with MAC addresses

ls /sys/class/net/        # Alternative listing

dmesg | grep -i eth       # Check kernel messages for NIC assignment

Match each interface to its VMware adapter by comparing the MAC address shown in "ip link show" with the MAC address displayed in VMware VM Settings for each network adapter. Update all configuration files to use the correct interface names.

## 7.6 Disabling Bridged Adapters After Setup (Optional)

Once all packages are installed and configured, you can optionally disable the Bridged adapters to fully isolate the lab environment.

### Option A — Disable in VMware Workstation Pro

- Power off the VM (or use VM → Power → Shut Down Guest).

- Right-click the VM → Settings...

- Select the Bridged network adapter (Network Adapter 2 or 3).

- Either uncheck "Connected" and "Connect at power on" to disable it, or click Remove to delete it entirely.

- Click OK to save.

### Option B — Disable in the OS

Remove or comment out the bridged NIC from Netplan and apply:

# Edit the config and remove the ens36/ens37 (bridged) section

sudo nano /etc/netplan/00-installer-config.yaml

sudo netplan apply

Or bring it down temporarily:

sudo ip link set ens36 down    # Replace ens36 with your bridged NIC name

Note: After disabling the Bridged adapter, the VMs will no longer have internet access. Make sure all packages are installed before doing this.